Instructions for Viewing and Capturing OS-6 Data Output

Mac OS X

Power Supply

OS-6 board

OS-6 data cable

USB to Serial cable

Figure 1. Components

Install Software:

- Connect USB drive to Mac OS X computer and open EcoSensors folder.

- Install the 3rd-party drivers for the USB-to-Serial cable on the Mac OS X computer by clicking the file: PL2303_1.4.0.dmg.

- Install the 3rd-party data logging software on the Mac OS X computer by clicking the file: CoolTerm.dmg.

Hardware Setup:

- Connect power to the OS-6.

- Connect the data cable to the OS-6 by attaching the ground wire (black) and data wire (red) to the TB3 connector on the OS-6, as shown in Figure 2.

- Connect data cable to the USB-to-Serial Cable.

- Connect USB end of the USB-to-Serial Cable to the Mac OS X computer.

Run Software:

- Open CoolTerm.

- On the top menu, press “Connect”, as shown in Figure 3.

- Data will be displayed in 3 columns, each separated by a comma.

  - First column is ozone reading (in ppb)

  - Second column is temperature (in °C)

  - Third column is Relative Humidity (in %) (example shown in figure 4).

- To log data while CoolTerm is running (Figure 5),

  - Click on “Connection” at the top menu

  - Go to “Capture to Textfile”

  - Press “Start”

  - Define where the file should be stored on the computer and press “Save”

  - CoolTerm will now automatically log all data.

- To stop logging data,

  - Click on “Connection” at the top menu

  - Go to “Capture to Textfile”

  - Press “Stop”

Figure 2. Data and ground wires connected to TB3 on OS-6

Press ‘Connect’

Figure 3. Connect CoolTerm to Instrument

Figure 4. Example of instrument output: ozone (PPB), T(°C), RH(%)

Figure 5. Start logging data

Figure 6. Stop logging data